MATERIAL SAFETY DATA SHEET

SECTION 1.  PRODUCT IDENTIFICATION

PRODUCT NAME:  	Tungsten Hexafluoride

CHEMICAL NAME:  	Tungsten Hexafluoride	FORMULA:  WF6

SYNONYMS:  	Tungsten Hexafluoride

MANUFACTURER:	Air Products and Chemicals, Inc.

7201 Hamilton Boulevard

Allentown, PA 18195-1501

PRODUCT INFORMATION:	(800) 752-1597

MSDS NUMBER:  1099			REVISION:  6

REVIEW DATE:  June 1998			REVISION DATE:  June 1998

SECTION 2.  COMPOSITION / INFORMATION ON INGREDIENTS

Tungsten hexafluoride is sold as pure product >99%.

CAS NUMBER:  7783-82-6

EXPOSURE LIMITS:  Air Products uses ACGIH 3 ppm ceiling for hydrogen fluoride as TLV.

OSHA:  None established 	ACGIH:  None established 	NIOSH:  None established

SECTION 3.  HAZARD IDENTIFICATION

EMERGENCY OVERVIEW

Tungsten hexafluoride is a toxic, corrosive, nonflammable liquefied compressed gas packaged in cylinders under its own vapor pressure of 2.4 psig at 70 °F.  The gas is colorless but fumes white in moist air.  Hydrolyzes to very corrosive hydrofluoric acid on contact with moisture.  Can cause severe chemical burns if inhaled or upon skin contact.  Wear self-contained breathing apparatus (SCBA) and fully protective suit when entering release area.

EMERGENCY TELEPHONE NUMBERS

(800) 523-9374	Continental U.S., Canada, and Puerto Rico

(610) 481-7711	other locations

ACUTE POTENTIAL HEALTH EFFECTS:

ROUTES OF EXPOSURE:

EYE CONTACT:  Irritation and/or burns to the cornea that may lead to vision impairment or loss.

INGESTION:  Burns to the mouth, esophagus and stomach.  Systemic reaction may occur and can be fatal.

INHALATION:  Corrosive and extremely irritating to the upper and lower respiratory system.  Deep lung inflammation (chemical pneumonitis), bleeding (pulmonary hemorrhage), abnormal fluid build up in the lungs (pulmonary edema) and a systemic reaction are possible and can be fatal.  Appearance of symptoms may be delayed.

SKIN CONTACT:  Burns to affected areas that can cause deep tissue damage and possibly a systemic reaction which can be fatal.  Burns may not be immediately painful or visible.

POTENTIAL HEALTH EFFECTS  OF REPEATED EXPOSURE:

ROUTE OF ENTRY:  Ingestion, inhalation, eye or skin contact.

TARGET ORGANS:  Eyes, skin, airway, lungs, liver, kidney, heart and bone.

SYMPTOMS:  Chronic exposure may cause chronic bronchitis, asthma, and abnormal fluoride accumulation in bone structure (fluorosis) due to low level systemic absorption.

MEDICAL CONDITIONS AGGRAVATED BY OVEREXPOSURE:  May aggravate asthma, emphysema or other respiratory diseases.

CARCINOGENICITY:  Tungsten hexafluoride is not listed as a carcinogen or potential carcinogen by NTP, IARC or OSHA Subpart Z.

SECTION 4.  FIRST AID MEASURES

Prompt medical attention is required in all cases of exposure to tungsten hexafluoride.  Effects may be delayed.

EYE CONTACT:  While holding eyelids open, flush eyes with water until calcium gluconate solution is available.  Seek medical treatment immediately.  Trained personnel should administer 1% calcium gluconate solution by continuous drip.

INGESTION:  Do not induce vomiting.  Dilute acid by drinking water, several ounces of Milk of Magnesia, milk, Mylanta, or several vials of calcium gluconate.  Gastric lavage with lime (calcium oxide) water may be performed by a physician.

INHALATION:  Move exposed personnel to uncontaminated area.  Seek prompt medical attention.  If not breathing, give artificial respiration.  Mouth to mouth resuscitation is not recommended.  If breathing is difficult, give oxygen.  Continue with administration of oxygen while waiting for medical attention.  If airway obstruction occurs the placement of an artificial airway, by an emergency medical technician, may be necessary.  Trained personnel should administer 2.5% calcium gluconate by nebulizer with patient in sitting position.

SKIN CONTACT:  Flush with copious amounts of water until treatment is available.  Remove contaminated clothing.  With gloved hand apply 2.5% calcium gluconate gel to the burn area.  An alternative treatment is immersion in an iced solution of 0.2% Hyamine 1622 (benzethonium chloride) or 0.13% Zephiran (benzalkonium chloride).  If immersion is impractical, soaked compresses of the same solution should be applied to the area.  Immersion or compresses must be used continuously for two hours.  Compresses should be changed every two minutes.  Burns covering an area greater than eight square inches require immediate treatment by a physician.  A physician should be consulted for all exposures.

NOTES TO PHYSICIAN:  If pain persists after above topical treatments, it may be necessary to inject 5% aqueous calcium gluconate beneath, around and into the burn area.  This will more likely be necessary in the treatment of extensive burns or small burns where treatment has been delayed.

The patient should be observed for clinical symptoms of hypocalcemia following ingestion or inhalation or following extensive burns.  Serum calcium, potassium and magnesium determinations must be performed immediately and periodically to monitor for hypocalcemia and electrolyte imbalance.  EKGs should be done immediately and periodically to monitor for arrhythmia, hypocalcemia and hyperkalemia.

If additional information is needed call the Air Products’ emergency number (Section 3) or consult the Air Products’ Safetygram 29 “First Aid for Hydrogen Fluoride.”

SECTION 5.  FIRE FIGHTING MEASURES

FLASH POINT:	AUTOIGNITION:	FLAMMABLE RANGE:

Not applicable 	Not applicable 	Not applicable

EXTINGUISHING MEDIA:  Product is nonflammable and does not support combustion.  Use extinguishing media appropriate for surrounding fire.

SPECIAL FIRE FIGHTING INSTRUCTIONS:  Evacuate all personnel from area.  If possible without risk, move cylinders away from fire area.  Cool cylinders with water spray until well after fire is out.  Self- contained breathing apparatus (SCBA) required.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Cylinders exposed to high heat or flame may rupture violently.  Runoff from fire fighting may be contaminated; check pH.

HAZARDOUS COMBUSTION PRODUCTS:  None known

SECTION 6.  ACCIDENTAL RELEASE MEASURES

STEPS TO BE TAKEN IF MATERIAL IS RELEASED OR SPILLED:  Evacuate all personnel from affected area.  Increase ventilation to the release area and monitor hydrogen fluoride levels.  Large releases may require considerable downwind evacuation.  Use appropriate protective equipment.  If leak is from cylinder or cylinder valve call the Air Products’ emergency telephone number. If the leak is in the user’s system, close the cylinder valve, safely vent the pressure, and purge with an inert gas before attempting repairs.

DOT 1996 North American Emergency Response Guidebook suggested evacuation distances for a large release:

Initial:  500 feet in all directions

Downwind protection:	0.3 miles - day; 	1.2 miles - night

SECTION 7.  HANDLING AND STORAGE

STORAGE:  Storage area temperatures should not exceed 125 °F (52 °C) and should be free from combustible materials.  Storage should be away from heavily traveled areas and emergency exits.  Avoid areas where salt or other corrosive materials are present.  Secure cylinders in an upright position and store in a well ventilated area protected from the weather.  Valve protection caps and valve outlet seals should remain on cylinders not connected for use. Separate full from empty cylinders.  Avoid excessive inventory and storage time.  Use a first-in first-out system.  Keep accurate inventory records.  Visually inspect stored cylinders on a routine basis, at least weekly, for any indication of leakage or other problems.  Local codes may have special requirements for toxic gas storage.

HANDLING:  Do not drag, roll or slide cylinder.  Use a suitable handtruck designed for cylinder movement.  Never attempt to lift a cylinder by its cap.  Secure cylinders at all times while in use.  Use a pressure reducing regulator or separate control valve to safely discharge gas from cylinder.  Use a check valve to prevent reverse flow into the cylinder.  Never apply flame or localized heat directly to any part of the cylinder.  When preparing to connect cylinder for use, always loosen valve outlet seal slowly.  Once cylinder has been connected to process, open cylinder valve slowly and carefully.  If user experiences any difficulty operating the cylinder valve, discontinue use and contact supplier.  Never insert an object (e.g., wrench, screwdriver, etc.) into valve cap openings.  Doing so may damage valve causing a leak to occur.  Use an adjustable strap-wrench to remove over-tight or rusted caps.

SPECIAL PRECAUTIONS:  Most metals are corroded by tungsten hexafluoride in the presence of moisture.  Systems should be kept free of moisture.  Purge system with dry inert gas (i.e., helium or nitrogen) before tungsten hexafluoride is introduced and when system is out of service.

Carbon steel, stainless steel, nickel, Monel or copper are suitable materials of construction for use when no moisture is present.  Do not use brass or aluminum.  Hastelloy, platinum or gold offer good resistance to corrosion when moisture is present.  Kel-F or Teflon are the preferred gasket materials.  Glass and ceramics are corroded by tungsten hexafluoride.

Always store and handle compressed gases in accordance with Compressed Gas Association, Inc. (ph. 703-412-0900) pamphlet CGA P-1, Safe Handling of Compressed Gases in Containers.  Local regulations may require specific equipment for storage or use.

SECTION 8.  EXPOSURE CONTROLS/PERSONAL PROTECTION

ENGINEERING CONTROLS:

VENTILATION:  Provide adequate ventilation and/or local exhaust to prevent accumulation of tungsten hexafluoride concentrations.

RESPIRATORY PROTECTION:

Emergency Use:  Self-contained breathing apparatus (SCBA) or a combination supplied air respirator equipped with an escape SCBA.

EYE PROTECTION:  Safety glasses for handling cylinders.  A full faceshield should be worn in addition to safety glasses when connecting, disconnecting or opening cylinders.

SKIN PROTECTION:  Leather gloves for handling cylinders.  Acid resistant gloves and splash suit when connecting, disconnecting or opening cylinders.  Encapsulating chemical protective suit for emergencies.

OTHER PROTECTIVE EQUIPMENT:  Safety shoes, safety shower, eyewash fountain.

SECTION 9.  PHYSICAL AND CHEMICAL PROPERTIES

APPEARANCE, ODOR AND STATE:  A colorless gas with a sharp, suffocating, pungent, acidic odor.

MOLECULAR WEIGHT:  297.8

BOILING POINT (1 atm):  62.7 °F (17.2 °C)

SPECIFIC GRAVITY (also called vapor density) (Air =1):  10.8

FREEZING POINT / MELTING POINT:  35.6 °F (2.0 °C)

VAPOR PRESSURE (At 70 °F (21.1 °C)):  2.7 psig

GAS DENSITY (At 70°F (21.1°C) and 1 atm):  0.8 lb/ft3

SOLUBILITY IN WATER (Vol./Vol. at 32 °F (0 °C) and 1 atm):  hydrolyzes

SECTION 10.  STABILITY AND REACTIVITY

CHEMICAL STABILITY:  Stable

CONDITIONS TO AVOID:  Cylinders should not be exposed to temperatures in excess of 125 °F (52 °C).

INCOMPATIBILITY (Materials to Avoid):  Moisture, aluminum and brass.

REACTIVITY:

A)  HAZARDOUS DECOMPOSITION PRODUCTS:  Hydrofluoric acid and tungsten oxyfluorides when hydrolyzed.

B)  HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 11.  TOXICOLOGICAL INFORMATION

LC50 (Inhalation):  217ppm (rat, 1 hour) (Estimated value from CGA Pamphlet P-20)

LD50 (Oral):  Not available

LD50 (Dermal):  Not available

CARCINOGENICITY:  No data

SKIN CORROSIVITY:  Tungsten hexafluoride can cause severe burns that may not be immediately painful or visible.

ADDITIONAL NOTES TO PHYSICIAN:  None

SECTION 12.  ECOLOGICAL INFORMATION

AQUATIC TOXICITY:  Tungsten hexafluoride is not listed as a marine pollutant by DOT(49CFR).  There is no definitive aquatic toxicity data available.

MOBILITY:  Unknown

PERSISTENCE AND BIODEGRADABILITY:  Unknown

POTENTIAL TO BIOACCUMULATE:  Unknown

REMARKS:  Do not release large amounts of tungsten hexafluoride to the atmosphere.  This product does not contain any Class I or Class II ozone depleting chemicals.

SECTION 13.  DISPOSAL CONSIDERATIONS

UNUSED PRODUCT / EMPTY CONTAINER:  Return cylinder and unused product to supplier.  Do not attempt to dispose of unused product.

DISPOSAL INFORMATION:  Scrubbing via caustic is the most common method of disposal.

SECTION 14.  TRANSPORT INFORMATION

DOT SHIPPING NAME:  Tungsten Hexafluoride

Poison - Inhalation Hazard, Zone B

HAZARD CLASS:  2.3

IDENTIFICATION NUMBER:  UN2196

SHIPPING LABEL(s):  Poison Gas, Corrosive

PLACARD (When required):  Poison Gas

SPECIAL SHIPPING INFORMATION:  Cylinders should be transported in a secure position in a well ventilated vehicle.  Never transport in passenger compartment of a vehicle.  Ensure cylinder valve is properly closed, valve outlet cap has been reinstalled, and valve protection cap is secured before shipping cylinder.

CAUTION:  Compressed gas cylinders shall not be refilled except by qualified producers of compressed gases.  The filling and shipping of a compressed gas cylinder without the written consent of the owner is in violation of federal law (49 CFR 173.301).

NAERG #:  125

SECTION 15.  REGULATORY INFORMATION

U.S. FEDERAL REGULATIONS:

EPA - ENVIRONMENTAL PROTECTION AGENCY

CERCLA:  Comprehensive Environmental Response, Compensation, and Liability Act of 1980

(40 CFR Parts 117 and 302)

Reportable Quantity (RQ):  None

SARA TITLE III:  Superfund Amendment and Reauthorization Act

SECTIONS 302/304:  Emergency Planning and Notification (40 CFR Part 355)

Extremely Hazardous Substances:  Tungsten hexafluoride is not listed.

Threshold Planning Quantity (TPQ):  None

Reportable Quantity (RQ):  None

SECTIONS 311/312:  Hazardous Chemical Reporting (40 CFR Part 370)

IMMEDIATE HEALTH:	Yes	PRESSURE:	No

DELAYED HEALTH:	No	REACTIVITY:	Yes

FIRE:	No

SECTION 313:  Toxic Chemical Release Reporting (40 CFR Part 372)

Tungsten hexafluoride does not require reporting under Section 313.

CLEAN AIR ACT:

SECTION 112 (r):  Risk Management Programs for Chemical Accidental Release

(40 CFR PART 68)

Tungsten hexafluoride is not listed as a regulated substance.

Threshold Planning Quantity (TPQ):  None

TSCA:  Toxic Substance Control Act

Tungsten hexafluoride is listed on the TSCA inventory.

OSHA - OCCUPATIONAL SAFETY AND HEALTH ADMINISTRATION:

29 CFR Part 1910.119:  Process Safety Management of Highly Hazardous Chemicals

Tungsten hexafluoride is not listed in Appendix A as a highly hazardous chemical. Threshold Planning Quantity (TPQ):  None

STATE REGULATIONS:

CALIFORNIA:

Proposition 65:  This product is not a listed substance which the State of California requires warning under this statute.

SECTION 16.  OTHER INFORMATION

NFPA RATINGS:	HMIS RATINGS:

HEALTH:	=  4	HEALTH:	=  2

FLAMMABILITY:	=  0	FLAMMABILITY:	=  0

REACTIVITY:	=  1	REACTIVITY:	=  1

SPECIAL: